# Scenario-Based Questionnaire: A Case Study of the Drilling-Reaming process

Definition of Hole Specification and Process Scenario Parameters

The hole specification and machining parameters define the functional requirements of the machined feature and the baseline operating conditions used to construct the scenario-based questionnaire and training dataset for the SimuDec framework.

Hole Specification:

- Hole Dimension = ∅10 x 30 mm

- Diameter tolerance = ±0.01 mm

- Depth tolerance = ±0.05 mm

- Roundness requirement = <0.01 mm

- Surface Roughness = <3.2 µm

Process Scenario (Baseline Machining Conditions):

- Process: Drill & Reaming

- Material: Gray Cast Iron (FC250)

- Drill bit: ∅ 9.8 x 87 x 133 mm (HSS: High Speed Steel)

- Reamer: ∅ 10 mm, flute 66 mm, OAL 133 mm

Cutting Velocity and Spindle Speed

- Reaming Cutting Velocity = 15 m/min

Corresponds to an approximate spindle speed of 475 rpm for the reamer.

- Drilling Cutting Velocity = 25 m/min

Corresponds to an approximately spindle speed of 790 rpm for the drill bit.

The spindle speed is calculated using the standard machining relationship:

v=πdn1000

Where:

v=cutting velocity (mm)

d=tool diameter (mm)

n=spinndle speed (rpm)

Table 1: Scenario dataset used in the drilling-reaming questionnaire

| ScenarioID | Coolant flag (on=1/off=0) | Speed Drill (rpm) | Speed Ream (rpm) | Feed (mm/ rev) | Tool Used (Hr) | Tool Life (Hr) | PilotHoleDiameter (mm) | Runout (mm) | Tool Overhang (mm) | Defect type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S01 | 1 | 600 | 200 | 0.1 | 10 | 1000 | 9.7 | 0.01 | 20 | Poor surface finish |
| S02 | 1 | 600 | 200 | 0.15 | 500 | 1000 | 9.8 | 0.03 | 40 | Poor roundness |
| S03 | 1 | 600 | 200 | 0.2 | 900 | 1000 | 9.9 | 0.06 | 60 | Poor roundness |
| S04 | 1 | 800 | 500 | 0.1 | 10 | 1000 | 9.8 | 0.03 | 60 | Chatter mark |
| S05 | 1 | 800 | 500 | 0.15 | 500 | 1000 | 9.9 | 0.06 | 20 | Oversize hole |
| S06 | 1 | 800 | 500 | 0.2 | 900 | 1000 | 9.7 | 0.01 | 40 | Poor surface finish |
| S07 | 1 | 1000 | 700 | 0.1 | 500 | 1000 | 9.7 | 0.06 | 40 | Poor roundness |
| S08 | 1 | 1000 | 700 | 0.15 | 900 | 1000 | 9.8 | 0.01 | 60 | Chatter marks |
| S09 | 1 | 1000 | 700 | 0.2 | 10 | 1000 | 9.9 | 0.03 | 20 | Undersize |
| S10 | 0 | 600 | 200 | 0.1 | 900 | 1000 | 9.9 | 0.03 | 40 | Scoring |
| S11 | 0 | 600 | 200 | 0.15 | 10 | 1000 | 9.7 | 0.06 | 60 | Chatter marks |
| S12 | 0 | 600 | 200 | 0.2 | 500 | 1000 | 9.8 | 0.01 | 20 | Poor surface finish |
| S13 | 0 | 800 | 500 | 0.1 | 500 | 1000 | 9.9 | 0.01 | 60 | Taper |
| S14 | 0 | 800 | 500 | 0.15 | 900 | 1000 | 9.7 | 0.03 | 20 | Poor surface finish |
| S15 | 0 | 800 | 500 | 0.2 | 10 | 1000 | 9.8 | 0.06 | 40 | Poor roundness |
| S16 | 0 | 1000 | 700 | 0.1 | 900 | 1000 | 9.8 | 0.06 | 20 | Scoring |
| S17 | 0 | 1000 | 700 | 0.15 | 10 | 1000 | 9.9 | 0.01 | 40 | Undersize |
| S18 | 0 | 1000 | 700 | 0.2 | 500 | 1000 | 9.7 | 0.03 | 60 | Chatter marks |

Questions

1. Place carefully review Scenario ID: 1, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer

- C2: Adjust feed speed

- C3: Adjust feed rate

- C4: Use coolant

- C5: Modify pilot hole size or drilling process

- C6: Correct runout/ alignment/ work holding

- C7: Reduce tool overhang

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer

- C2: Adjust feed speed

- C3: Adjust feed rate

- C4: Use coolant

- C5: Modify pilot hole size or drilling process

- C6: Correct runout/ alignment/ work holding

- C7: Reduce tool overhang

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

2. Place carefully review Scenario ID: 2, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer

- C2: Adjust feed speed

- C3: Adjust feed rate

- C4: Use coolant

- C5: Modify pilot hole size or drilling process

- C6: Correct runout/ alignment/ work holding

- C7: Reduce tool overhang

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer

- C2: Adjust feed speed

- C3: Adjust feed rate

- C4: Use coolant

- C5: Modify pilot hole size or drilling process

- C6: Correct runout/ alignment/ work holding

- C7: Reduce tool overhang

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

3. Place carefully review Scenario ID: 3, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

4. Place carefully review Scenario ID: 4, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

5. Place carefully review Scenario ID: 5, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

6. Place carefully review Scenario ID: 6, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

7. Place carefully review Scenario ID: 7, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

8. Place carefully review Scenario ID: 8, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

9. Place carefully review Scenario ID: 9, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

10. Place carefully review Scenario ID: 10, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

11. Place carefully review Scenario ID: 11, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

12. Place carefully review Scenario ID: 12, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

13. Place carefully review Scenario ID: 13, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

14. Place carefully review Scenario ID: 14, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

15. Place carefully review Scenario ID: 15, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

16. Place carefully review Scenario ID: 16, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

17. Place carefully review Scenario ID: 17, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency

18. Place carefully review Scenario ID: 18, including the process parameters and requirement specifications (tolerances, roundness, surface roughness).

Section 1: Root Cause Analysis

Q1. What is the most likely primary cause of the incomplete reaming in this scenario?

(Select One)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

Q2. Is there a secondary contributing cause?

(Select One or None)

- A1: Tool wear/ tool condition.

- A2: Chip packing/ chip evacuation problem.

- A3: Inappropriate spindle speed.

- A4: Inappropriate feed rate.

- A5: Coolant not used.

- A6: Improper pilot hole (diameter/condition).

- A7: Runout/ alignment/ work holding issue.

- A8: Excessive tool overhang.

- None (No secondary cause).

Q3. How confident are you in your root cause analysis?

(Select One)

- 1- Very Low

- 2- Low

- 3- Medium

- 4- High

- 5- Very High

Section 2: Decision Making

Q4. Based on the given requirements.

(Diameter tolerance, depth tolerance, surface roughness),

what decision do you recommend for this lot?

(Select One)

- D1: Rework

- D2: Scarp

- D3: Hold/further investigation

- D4: Accept as-is

Section3: Corrective Action

Q5. What is the primary corrective action you recommend?

(Select One)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

Q6. Is there a secondary corrective action you would recommend?

(Select One or None)

- C1: Replace or regrind reamer.

- C2: Adjust feed speed.

- C3: Adjust feed rate.

- C4: Use coolant

- C5: Modify pilot hole size or drilling process.

- C6: Correct runout/ alignment/ work holding.

- C7: Reduce tool overhang.

- C8: Improve chip evacuation method (e.g., intermittent withdrawal/ pecking)

- None

Section4: Effectiveness and Urgency Assessment

Q7. How effective do you expect the selected corrective action(s) to be in reducing this defect?

(Select One)

- 1: Less than 50%

- 2: 50-70%

- 3: 70-90%

- 4: More than 90%

Q8. How urgent is it to implement corrective action for this issue?

(Select One)

- 1: Not urgent

- 2: Low urgency

- 3: Medium urgency

- 4: High urgency

- 5: Very high urgency